| ## CONTROL CODES LIST |  |  |  |
| --- | --- | --- | --- |
| Control Code | Hexadecimal code | Function | Note |
| <LF> | 0A | Line feed |  |
| <CR> | 0D | Carriage return |  |
| <FF> | 0C | Form feed |  |
| <ESC> ”$” <n1> <n2> | 1B 24 n1 n2 | Set absolute print position | 1≤n1,n2≤30 n2>n1 |
| <ESC> “-“ “1” <ESC> “-“ <1> | 1B 2D 31 1B 2D 01 | Select underline mode |  |
| <ESC> “-“ “0” <ESC> “-“ <0> | 1B 2D 30 1B 2D 00 | Cancel underline mode |  |
| <ESC> “@” | 1B 40 | Initialize printer |  |
| <ESC> “J” | 1B 4A | Print and feed paper |  |
| <ESC> “d”<n> | 1B 64 n | Print and feed n lines | 1<n≦30 |
| <ESC> “p” | 1B 70 | Generate pulse |  |
| <ESC> “R” <n> | 1B 52 n | Select an international character | 1≦n≦10 Default n=0 Ref. to Table (1) |
| <ESC> “W” “1” <ESC> “W” <1> | 1B 57 31 1B 57 01 | Select double width character |  |
| <ESC> “W” “0” <ESC> “W” <0> | 1B 57 30 1B 57 00 | Cancel double width character |  |
| <ESC> “” | 1B 1E | Select double width character |  |
| <ESC> “” | 1B 1F | Cancel double width character |  |
| <BEL> | 07 | Generate pulse |  |

Remark:“ “ : ASCII code

< > : Hex   code

Ref. Table(1)  Select an international character set (Default n=0)

| n | ### Character Set |
| --- | --- |
| 0 | U.S.A. |
| 1 | France |
| 2 | Germany |
| 3 | U.K. |
| 4 | DenmarkⅠ |
| 5 | Sweden |
| 6 | Italy |
| 7 | Spain |
| 8 | Japan |
| 9 | Norway |
| A | DenmarkⅡ |

LF

LF [Print and Line feed]

LF 列印後跳行.

˙Programming Sample

1 PRINT #1,”AAAAA”;CHR$(&HA); 2 PRINT #1,”BBBBB”;CHR$(&HA);

˙Print Sample

| STEP | RESULT |
| --- | --- |
| 1 | ###### AAAAA |
| 2 | BBBBB |

BEL

BEL [Generate Pulse]

BEL 印表機輸出錢櫃控制訊號.

˙Programming Sample        PRINT #1,CHR$(07);

第九章

#### CR

CR [Carriage return]

CR 將印字位置設於起始位置

˙Programming Sample

1 PRINT #1,”0123456789”;CHR$(&HA); 2 PRINT #1,”AB”;CHR$(&HA); 3 PRINT #1,”0123456789”; 4 PRINT #1,CHR$(&HD); 5 PRINT #1,”AB”;CHR$(&HA);

˙Print Sample

| STEP | RESULT |
| --- | --- |
| 1 | ###### 0123456789 |
| 2 | ###### AB |
| 3,4,5 | AB23456789 |

說明:於Step 5 的0A碼送出才會列印 Line 3.

FF

FF [Form feed]

FF 跳到下一頁

˙Programming Sample

1 PRINT #1,”AAAAA”;CHR$(&HA); 2 PRINT #1,”BBBBB”;CHR$(&HA); 3 PRINT #1,CHR$(&HC); 4 PRINT #1,”CCCCC”;CHR$(&HA); 5 PRINT #1,”DDDDD”;CHR$(&HA);

˙Print Sample

| STEP | RESULT |
| --- | --- |
| 1 | ###### AAAAA |
| 2 | ###### BBBBB |
| 3 | <跳頁> |
| 4 | ###### CCCCC 下一頁延長起始位置 |
| 5 | ###### DDDDD |

第九章

ESC $ n m

ESC $ [Set absolute print position]

ESC $ 設定列印範圍在n與m之間.

˙Programming Sample

1 PRINT #1,CHR$(&H1B);”$”;CHR$(&HA);CHR$(&HE); 2 PRINT #1,”AAAAA”;CHR$(&HA); 3 PRINT #1,”BBBBB”;CHR$(&HA);

n=10(&HA) , m=14(&HE)

˙Print Sample

| STEP | RESULT |  |  |
| --- | --- | --- | --- |
| 1 | ###### Line1 |  |  |
|  | ###### Line2 |  |  |
|  | : |  |  |
|  | ###### : |  |  |
| 2 | ###### Line10 AAAAA |  |  |
| 3 | ###### Line11 BBBBB |  |  |
|  | ###### Line12 列印範圍 |  |  |
|  | Line13 |  |  |
|  | ###### Line14 |  |  |

說明:所有列印,設定在第10-14行之間.

遇到0C時自動跳到下一頁的第n行

,於此例會跳到下一頁的第10行.

注意:power off 之後,會被Reset掉.

ESC - n

ESC – n [Select/cancel/ underline mode]

ESC – n 將後續列印之內容,設定/取消底線列印.

˙Programming Sample

1 PRINT #1,CHR$(&H1B);CHR$(&H2D);”0”;”AA”; 2 PRINT #1,CHR$(&H1B);CHR$(&H2D);”1”;”BB”; 3 PRINT #1,CHR$(&H1B);CHR$(&H2D);”0”;”CC”; 4 PRINT #1,CHR$(&H1B);CHR$(&H2D);”1”;”DD”;CHR$(&HA); 5 PRINT #1,CHR$(&H1B);CHR$(&H2D);”30”;”EE”; 6 PRINT #1,CHR$(&H1B);CHR$(&H2D);”31”;”FF”; 7 PRINT #1,CHR$(&H1B);CHR$(&H2D);”30”;”GG”; 8 PRINT #1,CHR$(&H1B);CHR$(&H2D);”31”;”HH”; CHR$(&HA);

˙Print Sample

| STEP | RESULT |
| --- | --- |
| 1,2,3,4 | ###### AABBCCDD |
| 5,6,7,8 | ###### EEFFGGHH |

說明:使用Hex值,0與1結果與

Ascii值”0”與”1”結果相同.

第九章

ESC @

ESC @ [Initialize printer]

ESC @ 將印表機所有設定功能取消,重新設定為初始狀態.

˙Programming Sample

PRINT #1,CHR$(&H1B);”@”;

ESC J

ESC J [Print and feed paper]

ESC J 列印後印表機自動跳到下一頁.

˙Programming Sample

1 PRINT #1,”AAAAA”;CHR$(&HA); 2 PRINT #1,”BBBBB”;CHR$(&HA); 3 PRINT #1,CHR$(&H1B);”J”; 4 PRINT #1,”CCCCC”;CHR$(&HA); 5 PRINT #1,”DDDDD”;CHR$(&HA);

˙Print Sample

| STEP | RESULT |  |  |
| --- | --- | --- | --- |
| 1 | ###### AAAAA |  |  |
| 2 | ###### BBBBB |  |  |
| 3 | <跳頁> <- 下一頁起始位置(註) |  |  |
| 4 | ###### CCCCC |  |  |
| 5 | ###### DDDDD |  |  |

(註)若有ESC $ 指令時,則跳到下一頁設定起始位置.

第九章

ESC d n

ESC d [Print and feed n line]

ESC d 列印後印表機機自動跳n行.

1<n≦30

˙Programming Sample

1 PRINT #1,”AAAAA”;CHR$(&HA); 2 PRINT #1,”BBBBB”;CHR$(&HA); 3 PRINT #1,CHR$(&H1B);CHR$(&H64);CHR$(&H5); 4 PRINT #1,”CCCCC”;CHR$(&HA); 5 PRINT #1,”CCCCC”;CHR$(&HA);

˙Print Sample

| STEP | RESULT |
| --- | --- |
| 1 | ###### Line1 AAAAA |
| 2 | ###### Line2 BBBBB |
| 3 4 | Line3 Line4 Line5 Line6 Line7 Line8 CCCCC |
| 5 | ###### Line9 DDDDD |

ESC p

ESC p [Generate pulse]

ESC p 印表機輸出錢櫃控制訊號.

˙Programming Sample

說明:印表機若有街錢櫃時,控制訊號會將錢櫃打開.

PRINT #1,CHR$(&H1B);”p”

第九章

ESC R n

ESC R [Select an international character set]

ESC R 設定國碼.

˙Programming Sample

PRINT#1,CHR$(&H1B);”R”;CHR$(&H0); PRINT#1,”#$@[\]^’{¦}~”;CHR$(&HA);

說明:開機時初始設定值 n=0,參考 Ref.table(1)

˙Print Sample

#$@[\]^’{|}~

ESC W n

ESC W n [Select /concel Double width character mode]

ESC W n 將後續列印之內容,設定/取消雙倍字元寬度列印.

n=0/”0”,n=1/”1” ,0,1為Hex值,”0”,”1”為Ascii值

0,”0”=設定.   1,”1”=取消.

˙Programming Sample

1 PRINT #1,CHR$(&H1B);”W”;”0”;”AA”; 2 PRINT #1,CHR$(&H1B);”W”;”1”;”BB”; 3 PRINT #1,CHR$(&H1B);”W”;”0”;”CC”; 4 PRINT #1,CHR$(&H1B);”W”;”1”;”DD”;CHR$(&HA); 5 PRINT #1,CHR$(&H1B);”W”;”30”;”EE”; 6 PRINT #1,CHR$(&H1B);”W”;”31”;”FF”; 7 PRINT #1,CHR$(&H1B);”W”;”30”;”GG”; 8 PRINT #1,CHR$(&H1B);”W”;”31”;”HH”;CHR$(&HA);

˙Print Sample

| STEP | RESULT |
| --- | --- |
| 1,2,3,4 |  |
| 5,6,7,8 |  |

第九章

ESC  (▼)

ESC (▼)[Select/conceal Double width character mode]

ESC (▼)將後續列印之內容,設定/取消雙倍字元寬度列印.

˙Programming Sample

1 PRINT #1,CHR$(&H1B);”▼”;”AA”; 2 PRINT #1,CHR$(&H1B);””;”BB”; 3 PRINT #1,CHR$(&H1B);”▼”;”CC”; 4 PRINT #1,CHR$(&H1B);””;”DD”; 5 PRINT #1,CHR$(&H1B);”▼”;”EE”;CHR$(&HA);

˙Print Sample

說明:此指令功能與Esc w相同.

注意事項:

1．Power off 會reset所有設定.

2．在QBASIC下,若用LPRINT指令會自動加0D,0A

碼指令(依使用系統環境及程式語言有不同之情形).

3．若紙張起始位置,因人為裝紙發生錯誤時導致與

實際跳行數不符合時,會繼續列印到第27行.解

決這種問題的方法,將印表機電源關閉再開起並

將紙張起始位置調整好即可.

4．當印表機接收完一串資料後,最後一字元未跟隨

0A碼,若經5ms仍未傳送資料,會將上一串資料

的最後一行放棄不處理.(如:例1)

5．紙張起始定位時,建議使用0C指令及印表機上

的LF按鍵來定位(按住LF按超過1sec即可).

6．放大字型：本印表機只提供橫向放大兩倍字型

,於中文字可放８個字,英文字可放大１６個字.

7．中文字型處理：當中文字在無法全型列印時,會

自動跳到下一行.

例1:

PRINT #1,”AAAAA”;”CHR$(&HA);

PRINT #1,”12345”;

經5ms仍未送出0A碼時,則12345不處理.

結果只印出 AAAAA.